✅ PA‑32‑260 CHEROKEE SIX

## FLOW + VERIFY CHECKLIST (POH‑ALIGNED)

Aircraft: PA‑32‑260 Cherokee Six

Tail Number: N563VW

Engine: Lycoming O‑540‑E4B5

## 🛫 TAKEOFF

### ▶ FLOW

Fuel Pump — ON

Fuel Tank - FULLEST TANK

☐ Lights — AS REQUIRED

Flaps — SET 10° (25° SHORT/SOFT)

Fuel/Oil Gauges - GREEN

☐ Airspeed Alive — CHECK

☐ Rotate — 65-70 MPH

☐ Climb — Vy 105 MPH / Vx 95 MPH

🔵 INSTRUMENT FLIGHT RULES (IFR)

## ✈️ IFR — DEPARTURE / ENROUTE

### ▶ FLOW

☐ Clearance — REVIEW (route / altitude / frequencies)

☐ Bugs — HDG / ALT / MINs SET

☐ Navigation Source — CONFIRM

☐ Instruments — CHECK (gyro / CDI sensing)

☐ Fuel Pump — ON

☐ Lights — AS REQUIRED

## 🎯 IFR — APPROACH CONFIGURATION

### ▶ FLOW

☐ Power — SET

☐ Pitch — SET

☐ Configuration — FLAPS 10°

☐ Airspeed — 110 MPH

☐ Sink Rate — 500 FPM

☐ Garmin 530W approach activation — LOAD / VERIFY

☐ DADS — CHECK every 1 NM or 500 FT

☐ 6 Ts — APPLY / REVIEW on change

✅ Goal: Stabilized by FAF

## 📘 IFR PERFORMANCE PROFILES

| Phase | Power | Pitch | Config | Speed | V/S |
| --- | --- | --- | --- | --- | --- |
| Cruise | ~15 in MP | 0° | Clean | — | — |
| Descent | Low / Idle | −2 to −3° | Clean | 138 MPH | −500 fpm |
| Approach Level 1 | ~15 in MP | 0 to −1° | 25° | 104 MPH | −500 |
| Approach Level 2 | ~17 in MP | 0° | 33° | 104 MPH | −800 |
| Missed | FULL | +7° | Clean | 97 MPH | +800 |

## V SPEEDS

V1 - Abort Takeoff - 46 MPH (50% RWY)

Vs0 - stall w/ flaps - 63 MPH (55kts)

Vs1 - stall clean - 71 MPH (62kts)

Vr - rotate - 65 MPH (56kts)

Vy - best rate - 105 MPH (91kts)

Vx - best angle - 95 MPH (83kts)

BEST GLIDE - 100 MPH (87kts)

Vfe - flaps extend - 125 MPH (109kts)

Va - maneuvering - 149 MPH (129kts)

Vno - max cruise - 168 MPH (146kts)

Vne - never exceed - 212 MPH (184kts)

## 🛬 LANDING / GO‑AROUND

### Landing — FLOW

Downwind — 110 MPH

Abeam - 90-95 MPH

Over the #s - 80 MPH

☐ Flaps — AS REQUIRED (10° REC)

### Go‑Around — FLOW

⛔ Power — FULL

☐ Carb Heat — OFF

☐ Flaps — RETRACT IN STAGES

☐ Airspeed — ≥95 MPH

## 🚨 MISSED APPROACH

⛔ IMMEDIATE ACTIONS

Power — FULL

Pitch — +7°

Configuration — CLEAN

Airspeed — 97 MPH

Climb — +800 fpm

🟥 EMERGENCY & ABNORMAL OPERATIONS

## CLEAR RUNWAY

- Trim - TAKEOFF

- Flaps - UP

- Fuel Pump - OFF

- Landing Light - OFF

- Transponder - 1200

## SHUTDOWN

- Avionics - OFF

- Magnetos - CHECK

- Throttle - 1/4 INCH

- Mixture - FULL OUT/CUTOFF

- Magnetos - OFF

- Master - OFF

- Control Lock - SECURE

- Flight Plan - CLOSED

## 🚨 CRITICAL MEMORY ITEMS

(Always accessible)

☐ Radio call — PAN-PAN / MAYDAY as appropriate on 121.5

☐ Squawk — 7700

⛔ Engine Failure — 100 MPH

⛔ Engine Fire (In‑Flight) — Mixture IDLE CUTOFF

⛔ Electrical Fire / Smoke — MASTER OFF

🟩 NORMAL OPERATIONS

## 🧭 PREFLIGHT

## ▶ FLOW

Covers & Wheel Chalks - REMOVE

Paperwork - ARROW-AV1ATE

Control Lock - REMOVE

Flaps - DOWN

Trim - TAKEOFF

Master - ON

Lights - ON & CHECK

Pitot Heat - ON & CHECK

Stall Light - CHECK

Fuel Quantity — CHECK / PROPER TANK

Lights - OFF (KEEP BEACON ON)

Pitot Heat - OFF

Master - OFF

Airframe & Hinges - WALK AROUND & CHECK

Cowling - SECURE

Tires & Brakes - CHECK

Prop and Intake - CHECK

Fuel Tanks - SUMP, MEASURE

Oil — CHECK (6-10 QTS)

Tow Bar - STOW

Baggage - SECURE

## SAFETY CHECKLIST

- Seatbelts - EXPLAIN & ON

- Air vents/Heat - LOCATE & EXPLAIN

- First Aid & Fire Extinguisher - LOCATE

- Exits - EXPLAIN & LOCK

- Talking - ASSIST TRAFFIC

- Your Questions - ASK

## ENGINE START

Brakes - SET

Flaps - UP

Electrical - ALL OFF

Circuit Breaker - ALL IN

Carburetor Heat - OFF

Controls — FREE / CORRECT

Prop - FULL FORWARD

Mixture - RICH

Fuel Selector - STARTUP SIDE

Master - ON

Throttle - 1/4 INCH OPEN

Key/Mags - BOTH

Area - CLEAR PROP

Fuel Pump - ON

Primer - 2 TO 3 OR AS REQUIRED

Starter - CRANK ENGINE

Throttle - 1000 RPM

Avionics Master - ON

Fuel Pump - OFF

## GROUND CHECK

- Doors & Windows - CLOSED

- Seatbelts - SECURE

- Controls - CHECK

- Trim - TAKEOFF

- Airspeed Indicator - READS 0

- Attitude Indicator - <5° BANK AND WINGS LEVEL

- Altimeter - FIELD ELEVATION

- VSI - Zero

- Turn & Slip Indicator - FLAGS OFF, WINGS LEVEL, FULL FLUID

- Heading Indicator - SET TO COMPASS HEADING

☐ Garmin 530W database — CHECK / UPDATE

☐ Garmin 530W instrument check — VERIFY

## 🔍 RUN‑UP

### ▶ FLOW

☐ Brake — SET

☐ Throttle — 2000 RPM

☐ Magnetos — CHECK (175 RPM MAX/50 RPM DIFFERENCE)

☐ Propeller — CHECK / FULL FORWARD

Fuel/Oil Gauges — GREEN

Ammeter - POSITIVE AMPS

Suction — CHECK 5.0” +/- 1” HG

☐ Carburetor Heat — ON

Throttle — IDLE / SMOOTH

Carburetor Heat - OFF

Throttle - 800-1000 RPM

Flight Plan - ACTIVATE

### ✔ VERIFY

✔ Stable RPM

✔ No abnormal vibration

- Engine indications normal

## 🔥 ENGINE FAILURE — IN FLIGHT

### ▶ FLOW

☐ Radio call — MAYDAY, MAYDAY, MAYDAY on 121.5

☐ Squawk — 7700

⛔ Airspeed — 100 MPH (BEST GLIDE)

☐ Fuel Selector — FULLEST TANK

☐ Fuel Pump — ON

☐ Mixture — FULL RICH

☐ Carb Heat — ON

☐ Throttle — OPEN

### ✔ VERIFY

✔ Power restored → Resume normal operations

## 🔁 ENGINE RESTART — IN FLIGHT (POH)

### ▶ FLOW

☐ Fuel Selector — FULLEST

☐ Fuel Pump — ON

☐ Mixture — FULL RICH

☐ Carb Heat — ON

☐ Throttle — 1″ OPEN

☐ Magnetos — BOTH

☐ Primer — AS REQUIRED

☐ Engine Gauges — CHECK

## 🟫 FORCED LANDING

### ▶ FLOW

⛔ Airspeed — 100 MPH

☐ Landing Site — SELECT

☐ Mixture — IDLE CUTOFF

☐ Fuel Selector — OFF

☐ Fuel Pump — OFF

☐ Magnetos — OFF

☐ Master — OFF

☐ Seatbelts — SECURE

☐ Door — UNLATCH

☐ Flaps — AS REQUIRED

## 🔥 ENGINE FIRE — IN FLIGHT

### ▶ FLOW

☐ Radio call — MAYDAY, MAYDAY, MAYDAY on 121.5

☐ Squawk — 7700

⛔ Mixture — IDLE CUTOFF

☐ Fuel Selector — OFF

☐ Fuel Pump — OFF

☐ Magnetos — OFF

☐ Cabin Heat / Air — OFF

⛔ Airspeed — 125 MPH (TOP WHITE ARC)

➡ Proceed to Engine Failure / Forced Landing

## ⚡ ELECTRICAL FIRE / SMOKE (POH‑ALIGNED)

### ▶ FLOW

☐ Radio call — MAYDAY or PAN-PAN on 121.5

☐ Squawk — 7600 if comms fail; otherwise 7700

⛔ Master Switch — OFF

☐ All Electrical Switches — OFF

☐ Cabin Heat — OFF

☐ Air / Vents — OPEN (ventilate)

### ✔ VERIFY

✔ Smoke dissipating

### 🔄 IF ELECTRICAL POWER REQUIRED

☐ Master — ON

☐ Essential Circuits — ON ONE AT A TIME

⛔ If smoke returns:

➡ Master OFF → Land ASAP

## ⚠️ ALTERNATOR FAILURE / NO CHARGE

### ▶ FLOW

☐ Radio call — advise ATC / request assistance and vectors

☐ Squawk — as assigned; 7600 if radios fail

☐ Electrical Load — REDUCE

☐ Alternator Field CB — CHECK / RESET

☐ Alternator Output CB — CHECK / RESET

☐ Alternator — OFF (30 sec) → ON

### ✔ VERIFY

✔ Ammeter shows system load

⛔ If no output:

➡ Alternator OFF → Minimal load → Land ASAP / Practical

## IFR ACRONYMS / MNEMONICS

☐ IFR - Instrument Flight Rules

☐ VFR - Visual Flight Rules

☐ CDI - Course Deviation Indicator

☐ DME - Distance Measuring Equipment

☐ RNAV - Area Navigation

☐ ILS - Instrument Landing System

☐ FAF - Final Approach Fix

☐ MAP - Missed Approach Point

☐ MSA - Minimum Safe Altitude

☐ DA/MDA - Decision Altitude / Minimum Descent Altitude

☐ WAAS - Wide Area Augmentation System

☐ RAIM - Receiver Autonomous Integrity Monitoring

☐ ETE - Estimated Time Enroute

☐ ETA - Estimated Time of Arrival

☐ QNH - Altimeter setting to sea level

☐ DADS - Distance, Altitude, Direction, Speed. Check every 1 NM or 500 FT.

☐ 6 Ts - Turn, Time, Twist, Throttle, Talk, Track.

☐ GUMPF - Gas, Undercarriage, Mixture, Propeller, Flaps.

☐ GUMPS - Gas, Undercarriage, Mixture, Propeller, Seatbelts.

☐ CRAFT - Clearance, Route, Altitude, Frequency, Transponder.

☐ PAVE - Pilot, Aircraft, enVironment, External pressures.

☐ IMSAFE - Illness, Medication, Stress, Alcohol, Fatigue, Emotion.

☐ ARROW - Airworthiness certificate, Registration, Radio license, Operating handbook, Weight and balance.

☐ ATOMATOFLAMES - Common required instruments and equipment.

☐ 3 Ps - Perceive, Process, Perform.

☐ 5 Ps - Plane, Pilot, Passengers, Programming, Plan.